Readme

APM32F0xx SDK

Rev：V1.8

Introduction

The Geehy Semiconductor APM32F0xx software development kit includes a series driver library, a group of example applications that demonstrate key peripheral functionality, and other development files.

Software development kit have a hierarchy as follows:

- SDK directory

  - Boards

  - Documents

  - Examples

  - Libraries

  - Middlewares

  - Package

Table of Contents

1	Introduction	1

2	About boards	5

3	About documents	6

4	About examples	7

4.1	ADC_AnalogWindowWatchdog	11

4.2	ADC_ContinuousConversion	11

4.3	ADC_TMRTrigger	11

4.4	ADC_MultiChannelScan	12

4.5	ADC_VBAT	12

4.6	CAN_LoopBack	12

4.7	CEC_Communication	13

4.8	COMP_PWMSignalControl	13

4.9	CRC_CalcMessage	13

4.10	DAC_ADC	14

4.11	DAC_NoiseWave	14

4.12	DMA_ADC	14

4.13	DMA_Usart	15

4.14	DMA_MemoryToMemory	15

4.15	EINT	15

4.16	FMC_Write	16

4.17	GPIO_Toggle	16

4.18	I2C_TwoBoards	17

4.19	I2S_TwoBoards	17

4.20	IAP_Application1	18

4.21	IAP_Application2	18

4.22	IAP_BootLoader	18

4.23	IWDT_FeedDog	18

4.24	IWDT_FeedDog_Window	19

4.25	NVIC_WFI	19

4.26	PMU_WakeUp	20

4.27	RCM_ClockSwitch	21

4.28	RTC_Alarm	21

4.29	RTC_Calendar	22

4.30	RTC_Stamp	22

4.31	RTC_TimeStamp	23

4.32	RTC_LPWR_Wakeup	24

4.33	FreeRTOS	24

4.34	RT-Thread	25

4.35	RTX	25

4.36	SPI_FullDuplex	25

4.37	SPI_TwoBoards	26

4.38	SysTick	26

4.39	Template	27

4.40	TMR_6Steps	27

4.41	TMR_32BitCount	27

4.42	TMR_ComplementaryOutput	27

4.43	TMR_DMABurst	28

4.44	TMR_EncoderInterface	29

4.45	TMR_ExtTriggerSynchro	29

4.46	TMR_InputCapture	30

4.47	TMR_OCActive	31

4.48	TMR_OCInactive	31

4.49	TMR_OCToggle	32

4.50	TMR_ParallelSynchro	32

4.51	TMR_PWMInput	32

4.52	TMR_PWMOutput	33

4.53	TMR_SinglePulse	33

4.54	TMR_SynchronizationWithTMR1	33

4.55	TMR_TimeBase	34

4.56	TMR_TMRDMA	34

4.57	TSC_KeyLinearRotate	34

4.58	USART_Interrupt	35

4.59	USART_IrDA	35

4.60	USART_LIN	35

4.61	USART_Polling	36

4.62	USART_Smartcard	36

4.63	USBD_CDC	36

4.64	USBD_CUSTOM_HID_Keyboard	37

4.65	USBD_HID	37

4.66	USBD_MSC	37

4.67	USBD_WINUSB	37

4.68	WWDT_OverTime	38

5	About libraries	39

6	About middlewares	40

7	About Package	41

8	Revision History	42

About boards

The boards folder includes a board support package for APM32F0xx MINI board. It can help drive the peripheral circuit or components on the board quickly. The BSP can be found in the Boards directory.

The BSP provided are built for APM32F0xx MINI board compatibility. For other user development board use, some minor modifications may be required.

Boards have a hierarchy as follows:

- Boards folder

  - Board_APM32F030_MINI

  - Board_APM32F051_MINI

- Board_APM32F072_MINI

- Board_APM32F091_MINI

  - Board.c

  - Board.h

About documents

The documents folder includes a link file that can be redirected to the technical support center of Geehy semiconductor. The document can be found in the Documents directory.

About examples

The example applications can be found in the Examples directory.

The examples provided are built for APM32F0xx MINI board compatibility. For other user development board use, some minor modifications may be required.

Example projects have a hierarchy as follows:

- Example folder

  - Include

  - Project

    - IAR

    - Eclipse

    - MDK

  - Source

All example applications tested with: APM32F0xx StdPeriphDriver v1.0.4, include the following examples:

- Examples

  - ADC

    - ADC_AnalogWindowWatchdog

    - ADC_ContinuousConversion

    - ADC_TMRTrigger

    - ADC_MultiChannelScan

    - ADC_VBAT

  - CAN

    - CAN_LoopBack

    - CAN_Normal

- CEC

  - CEC_Communication

- COMP

  - COMP_PWMSignalControl

  - CRC

    - CRC_CalcMessage

  - DAC

    - DAC_ADC

    - DAC_NoiseWave

  - DMA

    - DMA_ADC

    - DMA_Usart

    - DMA_MemoryToMemory

  - EINT

    - EINT

  - FMC

    - FMC_Write

  - GPIO

    - GPIO_Toggle

  - I2C

    - I2C_TwoBoards

  - I2S

    - I2S_TwoBoards

  - IAP

    - IAP_Application1

    - IAP_Application2

    - IAP_BootLoader

  - IWDT

    - IWDT_FeedDog

    - IWDT_FeedDog_Window

  - NVIC

    - NVIC_WFI

  - PMU

    - PMU_WakeUp

  - RCM

    - RCM_ClockSwitch

  - RTC

    - RTC_Alarm

    - RTC_Calendar

    - RTC_Stamp

    - RTC_TimeStamp

    - RTC_LPWR_Wakeup

  - RTOS

    - FreeRTOS

    - RT-Thread

    - RTX

  - SPI

    - SPI_FullDuplex

    - SPI_TwoBoards

  - SysTick

    - SysTick

  - Template

    - Template

  - TMR

    - TMR_6Steps

    - TMR_32BitCount

    - TMR_ComplementaryOutput

    - TMR_DMABurst

    - TMR_EncoderInterface

    - TMR_ExtTriggerSynchro

    - TMR_InputCapture

    - TMR_OCActive

    - TMR_OCInactive

    - TMR_OCToggle

    - TMR_ParallelSynchro

    - TMR_PWMOutput

    - TMR_SinglePulse

    - TMR_SynchronizationWithTMR1

    - TMR_TimeBase

    - TMR_TMRDMA

- TSC

  - TSC_KeyLinearRotate

  - USART

    - USART_Interrupt

    - USART_IrDA

    - USART_LIN

    - USART_Polling

    - USART_Smartcard

  - USB

    - USBD_CDC

    - USBD_CUSTOM_HID_Keyboard

    - USBD_HID

    - USBD_MSC

    - USBD_WINUSB

  - WWDT

    - WWDT_OverTime

ADC_AnalogWindowWatchdog

Example Description

This example describes how to use ADC1 to monitor the voltage of ADC1_Channel0 continuously. When input Voltage of ADC1_Channel0(PA0) voltage is lower than 1.33v，LED2 is on, When input Voltage of ADC1_Channel0(PA0) voltage is higher than 1.66v，LED3 is on,

The converted voltage is displayed on serial assistant through USART1

Directory content

This example can be found in the ADC_AnalogWindowWatchdog directory.

ADC_ContinuousConversion

Example Description

This example describes how to use the ADC1 to convert continuously the voltage

applied to the APM32F030 or APM32F072 MINI ADC1_Channel0 input.

The converted voltage is displayed on serial assistant through USART1.

Directory content

This example can be found in the ADC_ContinuousConversion directory.

ADC_TMRTrigger

Example Description

This example describes how to use the ADC1 to convert continuously the voltage

applied to the APM32F030 or APM32F072 MINI ADC1_Channel0 input.

Each time the TMR1 CC4 event occurs, the ADC convert the variable voltage.

The converted voltage is displayed on serial assistant through USART1.

Directory content

This example can be found in the ADC_TMRTrigger directory.

ADC_MultiChannelScan

Example Description

This example describes how to use the ADC1 to scan continuously the voltage

applied to the APM32F072 MINI ADC1_Channel0 and ADC1_Channel1 and ADC1_Channel2 input.

The converted voltage is displayed on serial assistant through USART1..

Directory content

This example can be found in the ADC_MultiChannelScan directory.

ADC_VBAT

Example Description

This example describes how to use the ADC to convert VBAT voltage

applied to the APM32F072 MINI ADC_Channel 18.

The converted voltage is displayed on serial assistant through USART1.

Directory content

This example can be found in the ADC_VBAT directory.

CAN_LoopBack

Example Description

This example describes how to config a communication the CAN in loopback mode.

CAN transmit a message to self. Then compare the received message with transmitted

message. The LED2 turns on while the message is consistent, otherwise LED3 turns on.

The communication status will displayed on serial assistant through USART1.

Directory content

This example can be found in the CAN_LoopBack directory.

CEC_Communication

Example Description

Is example shows how to control CEC devices and communicate between two different boards.

To use this example, you need to load the same software into two APM32 boards (let's call them Master and Slave) then connect these two boards through CEC lines.

Directory content

This example can be found in the CEC_Communicationdirectory

COMP_PWMSignalControl

Example Description

This example shows how to use the comparator. COMP2 non-inverting input connect

to PA3. And COMP2 inverting input is internally connected to VREFINT(1.22V) which is

used to compare with PA3 input.

- External voltage should be connected to PA3.

- Connect an oscilloscope to TMR1 channel PA8 to display waveform.

- While PA3 is lower than VREFINT (1.22V), PWM signal is displayed on PA8.

- While PA3 is higher than VREFINT, PA8 is in high level.

Directory content

This example can be found in the COMP_PWMSignalControldirectory

CRC_CalcMessage

Example Description

- Write the calculated data to CRC DATA register and get the calculated result. The computer of serial debugging assistant can display the corresponding information for ComputedCRC.

Directory content

This example can be found in the CRC_CalcMessage directory.

DAC_ADC

Example Description

This example provides an example of how to use DAC channel 1(PA4) to output

voltage to ADC channel 10(PC0). The converted voltage of PA4 is detected by

ADC channel 10 and displayed on serial assistant through USART1.

-DAC generates voltage from 0V to 3.3V

-when the voltage is greater than 3.1V, then LED2 turns on and LED3 turns off.

-when the voltage is less than 0.8V, then LED2 turns off and LED3 turns on.

-when the voltage is greater than 0.8V but less than 3.1V, then LED2 and LED3 turn on.

Directory content

This example can be found in the DAC_ADC directory.

DAC_NoiseWave

Example Description

- This example shows how to configure the DAC peripheral to generate NoiseWaveThe waveform can be displayed using an oscilloscope. using DAC_CHANNEL_1(PA4) to output NoiseWave..

Directory content

This example can be found in the DMA_ NoiseWave  directory.

DMA_ADC

Example Description

This example provides an example of how to use a DMA channel to transfer continuously a data from a peripheral (ADC1) to DMA transfer. The ADC channel1 for APM32F030 or APM32F072 MINI Board is configured to be converted when device startup.

Directory content

This example can be found in the DMA_ADC directory.

DMA_Usart

Example Description

This example provides a basic communication between USART1 and USART2 using DMA1 capability.

After system reset, USART2 Transmit data from DMA_USART_TxBuf to USART1. Data received by USART1 is transferred by DMA and stored in DMA_USART_RxBuf. If the data of DMA_USART_TxBuf and DMA_USART_RxBuf are the same, LED2 and LED3 will constantly flicker alternately, or flicker together.

Directory content

This example can be found in the DMA_Usart directory.

DMA_MemoryToMemory

Example Description

This example shows how to configure the DMA peripheral to transmit data from

momery to momery. After system reset, data transmit form one group to another

through DMA. If the data received is equal to the data send, LED2 will light,

otherwise, LED3 will light..

Directory content

This example can be found in the DMA_ MemoryToMemory  directory.

EINT

Example Description

This example shows how to configure external interrupt lines. In this example, 2 EINT lines (KEY1、KEY2) when using the APM32F030 or APM32F072 MINI BOARD are configured to generate an interrupt on each falling edge. In the interrupt routine a led connected to a specific GPIO pin is toggled.

In this example

- EINT0 is mapped to PA0(KEY2)

- EINT1 is mapped to PA1(KEY1)

After EINT configuration：

- when falling edge is detected on EINT0, LED2 toggles

- when falling edge is detected on EINT1, LED3 toggles

Directory content

This example can be found in the EINT directory.

FMC_Write

Example Description

This example provides a description of how to program the flash address of APM32F030 or APM32F072.

After Reset, the Flash will be unlock. Then erase the specifies address and write a data in the address. In the end, lock the flash. The data of the address after erasing and programing will displayed on serial assistant through USART2.

Directory content

This example can be found in the FMC_Write directory.

GPIO_Toggle

Example Description

This example describes how to use DOUT for toggling IO. The IO of LED2 and LED3 is configed to toggle constantly. The phenomenon of LED2 and LED3 constantly flickered alternately.

Directory content

This example can be found in the GPIO_Toggle directory.

I2C_TwoBoards

Example Description

This example shows how to control I2C devices and communicate between two different boards.

- At startup, Boards master and slave are both in slave receiver mode and wait for messages to be received.

- When KEY1 is pressed on Board master, the master transmitter sent "Hello master" to Board slave. Message is displayed on serial assistant through USART2.

- When KEY1 is pressed on Board slave, the slave transmitter sent "Hello master" to Board master. The message is displayed on serial assistant through USART2.

Directory content

This example can be found in the I2C_TwoBoards directory.

I2S_TwoBoards

Example Description

This example provides a small application in which system sends and receives data by polling though using I2S firmware library. All received information will be displayed by serial assistant.

To use this example, you need to load it on two APM32F072 boards (let's call them Board master and Board slave). Then connect these two boards through I2S lines and must master and slave connect to the same GND.

After the program is downloaded, please reset the motherboard and then reset the slave. When compare buffer is the same, the Board slave LED2 is on. The phenomenon of data interaction process can be displayed using serial assistant.

Directory contents

This example can be found in the I2S_TwoBoards directory.

IAP_Application1

Example Description

This example shows how to generate a APP firmware to IAP.LED2 are toggled with a timing defined by the Delay function..

Directory contents

This example can be found in the IAP_Application1 directory.

IAP_Application2

Example Description

This example shows how to generate a APP firmware to IAP.LED3 are toggled with a timing defined by the Delay function.

Directory contents

This example can be found in the IAP_Application2 directory.

IAP_BootLoader

Example Description

The example aim to show how to configure a bootloader firmware to IAP.

When device connet to HyperTerminal right, a usart menu will show to user.

Directory contents

This example can be found in the IAP_ BootLoader  directory.

IWDT_FeedDog

Example Description

The example aim to show how to configure IWDT and feed dog to prevent a system reset. After IWDT initialization , System enters into infinite loop and feed dog within one second to prevent system reset. In the loop ,System will output information to serial assistant to display system status.

Directory contents

This example can be found in the IWDT_FeedDog directory.

IWDT_FeedDog_Window

Example Description

The example aim to show how to update the IWDG reload counter at special window period. After IWDT initialization, System enters into infinite loop and resets when feeding dog not in

window period. In the same time, System will output information to serial assistant to display

system status.

The IWDG counter is refreshed in the eint interrupt when press the KEY1 in window value to

prevent a IWDG reset.

Directory content

This example can be found in the IWDT_FeedDog_Window directory

NVIC_WFI

Example Description

This example describes how to use WFI event to enter sleep mode and wake up using external interrupt.

At startup, press KEY2(PA0) to occur Wait For Interrupt(WFI) event, and device will enter sleep mode. The device will wake up if press KEY2 again.

Directory content

This example can be found in the NVIC_WFI directory.

PMU_WakeUp

Example Description

This example shows how to enter the system by external interrupt to:

- STANDBY mode and wake-up from this mode either with the RESET or give PC13 a falling edge to recover.

- SLEEP mode and wake-up from this mode either with the RESET or give PA6 a falling edge to recover.

- STOP mode and wake-up from this mode with the RESET.

phenomenon:

-If press KEY1(SLEEP mode):  LED2 is on, while LED3 remain in the previous state. After giving PA6 a falling edge, system running from reset state.

-If press KET2(STANDBY mode): LED2 and LED3 are off. After giving PC13 a rising edge, system continue running.

-If low down PA7(STOP mode):  LED2 and LED3 remain in the previous state. After reset, system recover to normal state.

Directory content

This example can be found in the PMU_WakeUp directory.

RCM_ClockSwitch

Example Description

This example shows how to:

- Configure the PLL (clocked by HSE) as System clock source

- Configure HSI as System clock source

- Configure HSE as System clock source

- Output the System clock on MCO pin.

Through two buttons(KEY1、KEY2), switch the system clock.

phenomenon:

- If press KEY1(PLL as clock source): LED2 flashing quickly

- If press KEY2(HSI as clock source): LED2 flashing normally

- You can monitor the system clock on MCO pin (PA.8) or on serial assistant.

Directory content

This example can be found in the RCM_ClockSwitch directory.

RTC_Alarm

Example Description

This example shows how to configure RTC and ALARM.

phenomenon:

- After initialization, Alarm begin to count down with LED2 is on. Five second later, Alarm is waking up and LED2 is off.

- You can monitor the system state on serial assistant.

Directory content

This example can be found in the RTC_Alarm directory.

RTC_Calendar

Example Description

This example shows how to:

- Using RTC to set the system time and date.

- recover system time from reset state by using RTC_WriteBackup and RTC_ReadBackup function.

phenomenon:

- Time and date information are displayed through serial assistant.

- If reset (Power is on), system will read data from the backup area and display it.

- If reset (System Power from off to on), system will reinitialize the RTC.

Directory content

This example can be found in the RTC_Calendar directory.

RTC_Stamp

Example Description

This example shows how to write/read data to/from RTC Backup data registers and

demonstrates the Tamper detection feature.

The associated firmware performs the following:

1. It configures the Tamper pin to be falling edge, and enables the Tamper interrupt.

2. It writes the data to all RTC Backup data registers

3. On applying a low level on the RTC_AF1 pin (PC.13, connected to Tamper button),

the RTC backup data registers are reset and the Tamper interrupt is generated.

phenomenon:

- If Tamper1 interrupt happen, LED2 is on

- If Tamper2 interrupt happen, LED3 is on

- Time and Backup data are displayed in serial assistant.

Directory content

This example can be found in the RTC_Stamp directory.

RTC_TimeStamp

Example Description

This example provides a short description of how to use the RTC peripheral and

the Time Stamp feature.

The associated firmware performs the following:

1.It configures the RTC TimeStamp pin to be falling edge and enables the TimeStamp detection.

2.When RTC_TimeStamp pin is on the air, a interrupt will be generated and print TimeStamp information to serial assistant.

phenomenon:

- TimeStamp information is displayed in serial assistant.

Directory content

This example can be found in the RTC_TimeStamp directory.

RTC_LPWR_Wakeup

Example Description

This example shows how to configure RTC and ALARM .

phenomenon :

- After initialization, Alarm begin to count down with LED2 is on. Five second later, Alarm is waking up and LED2 is off.

- You can monitor the system state on serial assisatant..

Directory content

This example can be found in the RTC_LPWR_Wakeup directory.

FreeRTOS

Example Description

This example describes show how to how to use FreeRTOS create multiple tasks.

Usart test task : USART1 and USART2 send or received data to each other. Verification will occur after transmission,

if send and received data pass, LED3 will be on all the time.

if send and received data fault, LED3 will be off all the time.

if send or received data fault, LED3 will be constantly flickered alternately.

Led toggle task : The IO of LED2 is configed to toggle constantly

The phenomenon of LED2 constantly flickered alternately,

if Data transmission pass, LED3 will be on, and data interaction process can be displayed using serial assistant..

Directory content

This example can be found in the FreeRTOS directory.

RT-Thread

Example Description

This example describes how to use RT-Thread for APM32F0xx.

The IO of LED2 and LED3 is configed to toggle constantly.

The phenomenon of LED2 and LED3 constantly flickered alternately.

Directory content

This example can be found in the RT-Thread directory.

RTX

Example Description

This example describes show how to how to use RTX5 create multiple tasks.

Usart test task : USART1 and USART2 send or received data to each other. Verification will occur after transmission, if send and received data pass, LED3 will be on all the time. if send and received data fault, LED3 will be off all the time. if send or received data fault, LED3 will be constantly flickered alternately.

Led toggle task : The IO of LED2 is configed to toggle constantly

The phenomenon of LED2 constantly flickered alternately, if Data transmission pass, LED3 will be on, and data interaction process can be displayed using serial assistant.

Directory content

This example can be found in the RTX directory.

SPI_FullDuplex

Example Description

This demo is based on the APM32F030 or APM32F072 board, it shows how to use SPI peripheral. By making a master/slave full duplex communication between the SPI and the UART1.

The phenomenon of serial assistant can display information from USART2.

Directory content

This example can be found in the SPI_FullDuplex directory.

SPI_TwoBoards

Example Description

This example provides a small application in which system sends and receives data by polling

though using SPI firmware library. All received information will be displayed by serial assistant.

To use this example, you need to load it on two APM32F030 or APM32F072 boards (let's call

them Board master and Board slave). Then connect these two boards through SPI lines and

must master and slave connect to the same GND. When compare buffer is the same, the LED2 is on.

The phenomenon of data interaction process can be displayed using serial assistant.

Directory content

This example can be found in the SPI_TwoBoards directory.

SysTick

Example Description

This example shows how to configure the SysTick to generate a time base equal to

1 ms. The system clock is set to 8 MHz, the SysTick is clocked by the HSE clock.

A "Delay" function is implemented based on the SysTick end-of-count event.

For LEDs are toggled with a timing defined by the Delay function.

Directory content

This example can be found in the SysTick directory.

Template

Example Description

- This demo is based on the APM32F030/072 mini board. it provides a template project.

Directory contents

This example can be found in the Template directory.

TMR_6Steps

Example Description

The program to show how to configure the TMR1 peripheral to generate 6 Steps.

In this example, a software COM event is generated each 100 ms.

The TMR1 is configured in Timing Mode, each time a COM event occurs, a new TMR1

configuration will be set in advance..

Directory contents

This example can be found in the TMR_6Steps directory.

TMR_32BitCount

Example Description

This example describes how to configure the TMR3 and TMR15 realize the 32-bit timer.

TMR15 as High 16 bit count value, TMR3 as Low 16 bit count value. User can view the counter value through serial termainal.

Directory contents

This example can be found in the TMR_32BitCount directory.

TMR_ComplementaryOutput

Example Description

This example shows how to configure the TMR1 peripheral to generate complementary

TMR1 signals, to insert a defined dead time value.

The phenomenon of observe whether the output waveform is correct through

the oscilloscope. Besides LED2 and LED3 constantly flashing.

using TMR1 CH1 PA7 to output PWM

using TMR1 CH1N PA8 to output PWM

Directory contents

This example can be found in the TMR_ComplementaryOutput directory.

TMR_DMABurst

Example Description

The program to show how to configure the TMR1 channel period

and the duty cycle by DMA burst to generate 7 PWM

with 7 different duty cycles (80%, 70%, 60%, 50%, 40%, 30% and 20%).

On the DMA update request, the DMA will do 6 transfers of half words into TMR1

registers (AUTORLD, REPCNT, CC1, CC2, CC3, CC4).

999 will be transferred into AUTORLD

0 will be transferred into REPCNT

800 will be transferred into CC1.

700 will be transferred into CC2.

600 will be transferred into CC3.

500 will be transferred into CC4.

The objective is to generate 7 PWM signal at 1 KHz:

- SystemCoreClock = 48 MHz.

- TMR1CLK = SystemCoreClock, Prescaler =47, TMR1 counter clock = 1 MHz

- TMR1_Period = (1000000 / 1000) - 1

- Channel 1 duty cycle = 80%

- Channel 2 duty cycle = 70%

- Channel 3 duty cycle = 60%

- Channel 4 duty cycle = 50%

- Channel 3N duty cycle = 40%

- Channel 2N duty cycle = 30%

- Channel 1N duty cycle = 20%

Display TMR1 waveform by oscilloscope.

Directory contents

This example can be found in the TMR_DMABurst directory.

TMR_EncoderInterface

Example Description

This example describes how to configure the TMR1 peripheral to Encoder mode.

Directory contents

This example can be found in the TMR_EncoderInterface directory.

TMR_ExtTriggerSynchro

Example Description

This example shows how to synchronize TMR1 and TMR peripherals in cascade mode

with an external trigger

- TMR1 Master for TMR2 Config Toggle Mode and Enable event is used as Trigger Output

- TMR2 is slave for TMR1, And master for TMR3

- Config Toggle Mode and Enable event is used as Trigger Output

- The ITR0(TMR1) is used as input trigger

- TMR3 is slave for TMR2,

- Config Toggle Mode

- The ITR1(TMR2) is used as input trigger

- The TMR1 peripherals clock is 48 MHz.

- The Master Timer TMR1 frequency :

- TMR1 frequency = (48M)/ (23 + 1)/2 = 1MHz

- The TMR2 is running at:

- TMR2 frequency = (48M)/ (47 + 1)/2 = 500KHz

- The TMR3 is running at:

- TMR3 frequency = (48M)/ (95 + 1)/2 = 250KHz

The starts and stops of the TMR1 counters are controlled by the external trigger.

The TMR2 starts and stops are controlled by the TMR1, and the TMR3 starts and

stops are controlled by the TMR2.

Directory contents

This example can be found in the TMR_ExtTriggerSynchro directory.

TMR_InputCapture

Example Description

This example shows how to configure the TMR1 peripheral to capture the internal clock source from pin MCO.

The result will be displayed on serial assistant through USART2.

Directory contents

This example can be found in the TMR_InputCapture directory.

TMR_OCActive

Example Description

The program to show how to configure the TMR3 peripheral to generate 4 different

signals with four different delays.

- SystemCoreClock = 48MHz.

- TMR3CLK = SystemCoreClock = 48MHz.Prescaler = 47, TMR counter clock = 1 MHz

- TMR3_Period = (1000000 / 1000) - 1

- TMR3 CH1 pulse = 800

- TMR3 CH1 delay = 800/1MHz = 800us

- TMR3 CH2 pulse = 600

- TMR3 CH2 delay = 600/1MHz = 600us

- TMR3 CH3 pulse = 400

- TMR3 CH3 delay = 400/1MHz = 400us

- TMR3 CH4 pulse = 200

- TMR3 CH4 delay = 200/1MHz = 200us

The CHx delay correspond to the time difference between PE6 falling edge and  TMR3_CHx signal

rising edges. Reset system and display TMR3 waveform by oscilloscope.

Directory contents

This example can be found in the TMR_OCActive directory.

TMR_OCInactive

Example Description

The program to show how to configure the TMR3 peripheral in Output Compare Inactive

mode.

Directory contents

This example can be found in the TMR_OCInactive directory.

TMR_OCToggle

Example Description

The program to show how to configure the TMR3 peripheral to generate 4 waveform

with 4 different frequencies.

Directory contents

This example can be found in the TMR_OCToggle directory.

TMR_ParallelSynchro

Example Description

This example shows how to synchronize TMR peripherals in parallel mode.

Directory contents

This example can be found in the TMR_ParallelSynchro directory.

TMR_PWMInput

Example Description

This example describes how to use TMR3 Channel_2 (PE4 for APM32F072| PA7 for APM32F030) measure frequency and duty cycle of external signa. User can view the "DutyCycle" "Frequency "value through serial termainal.

TMR4_IRQHandler Function to calculate:

DutyCycle =  (TMR5_CC1*100)/(TMR5_CC2)  %.

Frequency = (RCM_ReadHCLKFreq()/2) / (TMR5_CC2) Hz.

The minimum frequency value to measure is 1 Hz..

Directory contents

This example can be found in the TMR_PWMInput directory.

TMR_PWMOutput

Example Description

This example shows how to configure the TMR1 peripheral to generate PWM signals

with different duty cycles. The TMR1 waveform can be displayed using an oscilloscope.

using TMR1 CHANNEL1(PA8) to output PWM

Directory contents

This example can be found in the TMR_PWMOutput directory.

TMR_SinglePulse

Example Description

This example shows how to use the TMR peripheral to generate a One-pulse Mode

after a falling edge of an external signal is received in Timer Input pin.

The TMR3 waveform can be displayed using an oscilloscope.

Directory content

This example can be found in the TMR_SinglePulse directory..

TMR_SynchronizationWithTMR1

Example Description

This example shows how to synchronize TMR peripherals in cascade mode, two timers TMR1 and TMR3 are used.

The phenomenon of observe whether the output waveform is correct through the oscilloscope.

Using TMR3 CH1(PA6) to output PWM

Directory content

This example can be found in the TMR_SynchronizationWithTMR1 directory.

TMR_TimeBase

Example Description

This example aims to show how to realize timing one second by using TMR14 peripheral generating time base.

LED2 and LED3 will toggle per second.

Directory content

This example can be found in the TMR_TimeBase directory.

TMR_TMRDMA

Example Description

This example aims to show how to realize timing one second by using TMR14 peripheral generating time base.

LED2 and LED3 will toggle per second.

Directory content

This example can be found in the TMR_TimeBase directory.

TSC_KeyLinearRotate

Example Description

This example provides a description of how use the TSC (Touch sensing controller) device on APM32F072 to achieve Key，Linear，Rotation control.

The five touch keys to control LED on and off.

The linear rotation touch will display the value on the OLDE.

Directory content

This example can be found in the TSC_KeyLinearRotate directory.

USART_Interrupt

Example Description

Through The computer of serial debugging assistant, display the message sent and received between the MCU and USART1.

The phenomenon of serial assistant can display information from USART1 and debug information of USART2.

Directory content

This example can be found in the USART_Interrupt directory.

USART_IrDA

Example Description

The program shows how to using USART IrDA mode, in this case,

USART1 sends data to upper computer. You can check the data in a Serial Port Utility.

Directory content

This example can be found in the USART_IrDA directory.

USART_LIN

Example Description

The program shows how to using USART LIN mode, in this case,

USART1 sends data to upper computer. You can check the data in a Serial Port Utility.

Directory content

This example can be found in the USART_ LIN  directory.

USART_Polling

Example Description

The program aims to show how to send or received data by using USART, in this case, USART1 and USART2 send or received data to each other. Verification will occur after transmission, if passed, LED2 will be on.

The phenomenon of data interaction process can be displayed using serial assistant.

Directory content

This example can be found in the USART_Polling directory.

USART_Smartcard

Example Description

The program shows how to using USART Smartcard mode, in this case,

USART1 sends data to upper computer. You can check the data in a Serial Port Utility.

Directory content

This example can be found in the USART_Smartcard directory.

USBD_CDC

Example Description

This example describes how to use USBD to simulate a CDC device.

Program will send hello + num string to USB host. And when CDC

device receive data will send back the same data to USB host.

Directory content

This example can be found in the USBD_CDC directory.

USBD_CUSTOM_HID_Keyboard

Example Description

This example describes how to use USBD to simulate a custom HID keyboard.

Press KEY1 will send the report descriptor of a - z or Enter to the USB host.

The state of Capslock and numlock determine whether LED2 and LED3 are on or off.

Directory content

This example can be found in the USBD_CUSTOM_HID_Keyboard directory.

USBD_HID

Example Description

This example describes how to use USB to simulate a HID mouse.

Press KEY1 will move the cursor to the left.

Press KEY2 will move the cursor to the right.

Directory content

This example can be found in the USBD_HID directory.

USBD_MSC

Example Description

This example describes how to use sram array to simulate a fake U disk.

Directory content

This example can be found in the USBD_MSC directory.

USBD_WINUSB

Example Description

This example describes how to use USBD to simulate a WINUSB device.

Program will send hello + num string to USB host. And when WINUSB

device receive data will send back the same data to USB host.

Directory content

This example can be found in the USBD_WINUSB directory.

WWDT_OverTime

Example Description

This example aims to show how to use WWDT.

If is_OverTime = 0, System would not reset for feeding dog timely. LED2 Toggle.

If is_OverTime = 1, System will reset. LED3 ON.

Directory content

This example can be found in the WWDT_OverTime directory.

About libraries

The libraries folder includes a series library. It can provide supports for APM32F0xx MCU such as device support and standard peripheral. The libraries can be found in the Libraries directory.

APM32F0xx MCU include following library:

- Libraries folder

  - APM32F0xx_StdPeriphDriver

  - CMSIS

  - Device

  - TSC_Device_Lib

About middlewares

The middlewares can be found in the Middlewares directory.

The middlewares used by APM32F0xx MINI include following:

- Middlewares folder

  - APM32_USB_Library

  - FreeRTOS

  - RealThread

  - RTX5

About Package

The Package folder includes Geehy APM32F0xx_DFP Package. The Package can be found in the Package directory.

The Package used by APM32F0xx MINI include following:

- Package folder

  - Geehy.APM32F0xx_DFP.1.1.0.pack

Revision History

Table 1 File Revision History

| Date | Rev | Description |
| --- | --- | --- |
| 2020.02.09 | 1.0 | First release version of APM32F0xx SDK V1.0 |
| 2020.02.09 | 1.1 | Update pack file. |
| 2020.03.11 | 1.2 | Update Example file. Update pack file. |
| 2020.11.30 | 1.3 | Add APM32F07x support files |
| 2021.04.26 | 1.4 | Add APM32F091 support files |
| 2021.07.01 | 1.5 | Update Library for V1.0.1. Update pack file for SVD. |
| 2022.02.21 | 1.6 | Update Library for V1.0.2. Update Pack for V1.0.7. Add IAR Support. Add USB_Device_Lib and Example. Add TSC_Device_Lib and Example. Add ADC_TMRTrigger and I2S_TwoBoards Example. |
| 2022.09.20 | 1.7 | Update Example file. |
| 2025.07.07 | 1.8 | Update USB Example. |

Statement

This document is formulated and published by Geehy Semiconductor Co., Ltd. (hereinafter referred to as “Geehy”). The contents in this document are protected by laws and regulations of trademark, copyright and software copyright. Geehy reserves the right to make corrections and modifications to this document at any time. Please read this document carefully before using Geehy products. Once you use the Geehy product, it means that you (hereinafter referred to as the “users”) have known and accepted all the contents of this document. Users shall use the Geehy product in accordance with relevant laws and regulations and the requirements of this document.

1. Ownership

This document can only be used in connection with the corresponding chip products or software products provided by Geehy. Without the prior permission of Geehy, no unit or individual may copy, transcribe, modify, edit or disseminate all or part of the contents of this document for any reason or in any form.

The “极海” or “Geehy” words or graphics with “®” or “TM” in this document are trademarks of Geehy. Other product or service names displayed on Geehy products are the property of their respective owners.

2. No Intellectual Property License

Geehy owns all rights, ownership and intellectual property rights involved in this document.

Geehy shall not be deemed to grant the license or right of any intellectual property to users explicitly or implicitly due to the sale or distribution of Geehy products or this document.

If any third party’s products, services or intellectual property are involved in this document, it shall not be deemed that Geehy authorizes users to use the aforesaid third party’s products, services or intellectual property, unless otherwise agreed in sales order or sales contract.

3. Version Update

Users can obtain the latest document of the corresponding models when ordering Geehy products.

If the contents in this document are inconsistent with Geehy products, the agreement in thesales order or the sales contract shall prevail.

4. Information Reliability

The relevant data in this document are obtained from batch test by Geehy Laboratory or cooperative third-party testing organization. However, clerical errors in correction or errors caused by differences in testing environment may occur inevitably. Therefore, users should understand that Geehy does not bear any responsibility for such errors that may occur in this document. The relevant data in this document are only used to guide users as performance parameter reference and do not constitute Geehy’s guarantee for any product performance.

Users shall select appropriate Geehy products according to their own needs, and effectively verify and test the applicability of Geehy products to confirm that Geehy products meet their own needs, corresponding standards, safety or other reliability requirements. If loses are caused to users due to the user’s failure to fully verify and test Geehy products, Geehy will not bear any responsibility.

5. Legality

USERS SHALL ABIDE BY ALL APPLICABLE LOCAL LAWS AND REGULATIONS WHEN USING THIS DOCUMENT AND THE MATCHING GEEHY PRODUCTS. USERS SHALL UNDERSTAND THAT THE PRODUCTS MAY BE RESTRICTED BY THE EXPORT, RE-EXPORT OR OTHER LAWS OF THE COUNTIRIES OF THE PRODUCTS SUPPLIERS, GEEHY, GEEHY DISTRIBUTORS AND USERS. USERS (ON BEHALF OR ITSELF, SUBSIDIARIES AND AFFILIATED ENTERPRISES) SHALL AGREE AND PROMISE TO ABIDE BY ALL APPLICABLE LAWS AND REGULATIONS ON THE EXPORT AND RE-EXPORT OF GEEHY PRODUCTS AND/OR TECHNOLOGIES AND DIRECT PRODUCTS.

6. Disclaimer of Warranty

THIS DOCUMENT IS PROVIDED BY GEEHY "AS IS" AND THERE IS NO WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, TO THE EXTENT PERMITTED BY APPLICABLE LAW.

GEEHY WILL BEAR NO RESPONSIBILITY FOR ANY DISPUTES ARISING FROM THE SUBSEQUENT DESIGN OR USE BY USERS.

7. Limitation of Liability

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW OR AGREED TO IN WRITING WILL GEEHY OR ANY OTHER PARTY WHO PROVIDE THE DOCUMENT "AS IS", BE LIABLE FOR DAMAGES, INCLUDING ANY GENERAL, SPECIAL, DIRECT, INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE DOCUMENT (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR DATA BEING RENDERED INACCURATE OR LOSSES SUSTAINED BY USERS OR THIRD PARTIES).

8. Scope of Application

The information in this document replaces the information provided in all previous versions of the document.

© 2022 - 2025 Geehy Semiconductor Co., Ltd. - All Rights Reserved